An Ordinance to Amend Chapter 201 (Streets and Sidewalks) of the Code

of the Town of Wells

to Expand Where Sidewalks are Required and Revise Sidewalk Requirements

NOTE:		Proposed additions to existing Code sections are underlined.

Proposed deletions of existing Code sections are crossed out.

Other sections of the Ordinance are unchanged.

The Town of Wells hereby ordains and enacts “An Ordinance to Amend Chapter 201 (Streets and Sidewalks) of the Code of the Town of Wells to Expand Where Sidewalks are Required and Revise Sidewalk Requirements” to read as follows:

#### Part 1:

#### § 201-38. Applicability.

This article shall be applicable only within the area of the Town that is covered by the Town of Wells Sidewalk Development Plan (2003) or the Route 1 Transportation Feasibility Study 2024 (Route 1, Chapel Road, College Drive, Sanford Road). Sidewalk improvements shall be designed in accordance with the standards established in the Town of Wells Sidewalk Development Plan or the Route 1 Transportation Feasibility Study (2024) and as specified in this article.

#### § 201-39. Deviations.

Deviations from these specifications may be obtained from the Road Commissioner if it is found necessary to ensure public safety. All deviations must receive approval prior to construction, or prior to approval of the development plan by the Planning Board, if applicable. The Road Commissioner may allow the design and construction of sidewalks that do not meet these standards upon a written finding that application of these standards would not promote the intent of the Sidewalk Development Plan or the Route 1 Transportation Feasibility Study, that physical conditions of a particular site do not allow for full implementation of these standards, and that safety of pedestrians and bicyclists will not be put at risk by following alternate design and construction standards.

#### § 201-40. (Reserved) Bike Lanes

#### To improve the safety of bicyclists and promote bicycle use versus increased vehicle traffic on Route 1, the Route 1 Transportation Feasibility Study recommends adding bike lanes on Route 1. The location of sidewalks may be impacted to allow for future bike lanes to be established. See Study for guidance on requirements, locations and construction standards.

#### § 201-41. Requirements for new sidewalk construction.

A. The following types of development in the Town shall provide for the location and construction of sidewalks as provided in this section.

(1) Establishment of a commercial use that requires construction of any new detached or attached building other than an accessory structure less than 1,000 square feet. other than a single-family dwelling or duplex on a vacant lot

(2) Establishment of a use that requires the demolition or alteration of an existing building, provided the alteration or demolition affects more than 75% of the gross floor area within the existing perimeter walls of the building. Construction of a new building or the demolition or alteration of an existing building on a lot that contained a building, provided the alteration affects more than 75% of the floor area or perimeter walls of the existing building, other than the establishment of a single-family dwelling or duplex on a vacant lot.

(3) The expansion of a structure other than a single-family dwelling or a duplex by more than 2,000 square feet of gross floor area after April 15, 2003 November 4, 2025.

(4) The conversion of a residential use to a commercial use, if the residential use is discontinued.

(5) The construction of a new dwelling unit.

B. This Development required to provide for the location and construction of sidewalks shall include the dedication of necessary right-of-way or public access easement and the construction of sidewalks in accordance with the specifications provided in § 201-40 and 42 or a contribution to the Town's Sidewalk Construction Fund as specified in § 201-43. Newly constructed sidewalks shall be paid for entirely by the property owner unless otherwise determined by the Planning Board. Provision for sidewalk construction shall be included as part of site plan review or subdivision approval and/or part of plans submitted for obtaining a building permit.

#### § 201-42. Sidewalk construction standards.

A. All sidewalks and related improvements including but not limited to curbs, pavement, and handicapped accessways, shall be constructed according to the standards in the Sidewalk Development Plan or the Route 1 Transportation Feasibility Study (2024), including those in this article.

B. Sidewalks shall be at least five feet wide and shall be located between the curb or grade line of the public street and the right-of-way line or public access easement if approved by the Town, but no closer than two feet to the curb or grade line. Sidewalk width and distance to curb or grade line may be reduced where right-of-way width or utilities does not allow this standard to be met. The Town may approve alteration of the alignment so the sidewalk meanders within the area between the curb and right-of-way line or public access easement. The Town may request that an easement be granted from property owners to locate the sidewalk if it is necessary for public safety and/or if the landowner is willing in order to increase green space between the roadway curb and the sidewalk.

C. Where a new sidewalk adjoins existing sidewalks that are not five feet in width, the new sidewalk shall taper on each side over a five-foot length to meet the existing condition.

D. A site plan indicating the location of the sidewalk shall be submitted to the appropriate Town reviewing authority determined by the Reviewing Authority Chart in § 145-71A for site plan approval or to the Planning Board for subdivision approval.

E. All sidewalks shall be constructed in accordance with the Americans with Disabilities Act (ADA) standards. Wheelchair access ramps must be constructed at any point a proposed sidewalk intersects a Town street with the exception of walks leading from the street to the door of a single-family residence. Access ramps shall be built to grades no greater than one foot of fall per 12 feet in length.

F. Sidewalk materials. Sidewalks shall be constructed of the materials specified in Table 5.4.1 of the Sidewalk Development Plan[1] in accordance with the details and specifications in the Sidewalk Development Plan.

G. Curb materials. Curb materials shall be constructed of the following materials in accordance with the specifications in the Sidewalk Development Plan.

(1) Vertical granite. (See Sidewalk Development Plan Figure 4.5.A.)

H. To the extent possible, sidewalks shall be constructed to within one foot of the right-of-way to maximize green space between the roadway curb or grade line.

#### § 201-43. Sidewalk Construction Fund. (Reserved)

A. The Town Treasurer shall establish a nonlapsing Sidewalk Construction Fund for the purpose of financing the construction and improvement of sidewalks in the area covered by the Sidewalk Development Plan. The Sidewalk Construction Fund shall be segregated from the municipality's general revenues.

B. The Sidewalk Construction Fund may be used provide the local match to state and federal grants for sidewalk construction and improvement.

C. Funds from the Sidewalk Construction Fund shall not be used for any purpose other than the construction and improvement of sidewalks in the area covered by the Sidewalk Development Plan.

D. A property owner in the area covered by the Sidewalk Development Plan may, in lieu of construction of a sidewalk as otherwise required by § 201-42, make a contribution to the Sidewalk Development Fund.

E. The amount of the required contribution shall be calculated by multiplying the average per foot cost for construction of the segment by the length of sidewalk adjacent to or on the property.

F. At the time a contribution to the Sidewalk Construction Fund is made, the Town shall record the date, amount, name and address of the contributor, and the assessor's map and lot number of the property for which the contribution is made. If within seven years of the date of the contribution, the Town has not constructed the segment of sidewalk for which a contribution has been made to the Fund or included such construction in the budget for the current fiscal year, then the contribution shall be refunded to the owner of record of the property from which the contribution was credited.

G. If the town receives state or federal funding for the construction of a sidewalk, then the town shall refund to the owner of record of the property from which the contribution was credited an amount proportional to the funding received from state or federal sources.

Part 2: Effective Date.

This Ordinance shall take effect upon adoption by the Town Meeting.

Given under our hands this ____ day of ______________, 2025.

THE SELECT BOARD OF THE TOWN OF WELLS:

_____________________________		_______________________________

John MacLeod III				Avery Seuter

_____________________________		_______________________________

Kathleen Chase				Timothy Roche

_______________________________

James N. Smith